MULTI CYCLONE SEPARATOR- High-Efficiency Parallel Cyclone Battery System for Fine Dust Collection

# Product Overview:

Multi Cyclone Separator consists of multiple small-diameter cyclones housed in a single chamber operating in parallel. Small diameter creates higher centrifugal force, enabling collection of fine particles down to 5-10 microns with 90-95% efficiency. It acts as a pre-cleaner before bag filters/ESPs or as final collector for coarse/medium dust.

Ideal For:

- Boiler flue gas

- Cement plants

- Rice mills

- Plywood

- Thermal power

- Sugar mills

Key Features:

- Multiple Small Cyclones: 6-200 parallel tubes of 150-400mm dia give higher “g” force than single large cyclone. Collects finer dust.

- High Collection Efficiency: 90-95% for PM10. Reduces load on downstream bag filter by 80-90%

- Low Pressure Drop: Optimized tube design keeps pressure drop 100-200 mmWC, saving fan power vs wet scrubbers.

- Compact Design: Handles large gas volumes in small footprint vs conventional cyclones.

- No Moving Parts: Zero consumables. Only hopper discharge system needs maintenance.

- Abrasion Resistant: Ni-hard/ceramic liners available for highly erosive dust like boiler fly ash.

- Modular Construction: Damaged tubes can be plugged individually without stopping entire unit.

- Dry Collection: Collected dust is dry, can be reused/sold. No water pollution.

# Technical Specifications:

- Capacity Range: 1,000 CMH to 3,00,000 CMH

- Type of Fuel: Coal, Biomass, Pet Coke, Wood Chips & Process Dust Applications

- Temp: Up to 300°C depending on design and lining

- Types of Separation: Multi Tube / High Efficiency / Tangential Cyclone Type

- Types of Discharge: Rotary Airlock / Screw Conveyor / Double Dump Valve

- MOC: Carbon Steel (IS2062) / SS304 / SS316