WET SCRUBBER- Air Pollution Control Device for Gas Cleaning & Particulate Removal

Product Overview:

Wet Scrubber is an air pollution control system that removes particulates and soluble gases from industrial exhaust streams by contacting dirty gas with a scrubbing liquid, typically water. It handles sticky, corrosive, high-temperature, and explosive dusts where dry collectors like bag filters are unsuitable.

Ideal For:

- Chemical fumes

- Acid mists

- Foundry dust

- Boiler flue gas

- Odorous gases

- Combustible dusts

Key Features:

- Dual Removal: Simultaneously removes particulate matter and absorbs gaseous pollutants like SO₂, HCl, NH₃ and Cl₂.

- Handles Tough Dust: No blinding or choking with sticky, hygroscopic, or oily dust. Fire/explosion risk eliminated.

- High Temp Handling: Hot gases directly taken without pre-cooling. Quenching section reduces temp before main scrubbing.

- Corrosion Resistant: PP/FRP/rubber lining used for acidic or alkaline gases. No corrosion issues like in bag filters.

- Compact Design: Lower footprint than ESPs for same gas volume. Suitable for space constraints.

- Low Maintenance: No bags or electrodes to replace. Only pump, nozzles, and pH dosing need attention.

- Turndown Capability: Can operate at 50-110% of design flow with stable efficiency.

- Byproduct Recovery: Scrubbing liquid can be treated to recover product or neutralize for safe disposal.

Technical Specification:

- Capacity Range: 500 CMH to 3,00,000 CMH

- Type of Fuel: Coal, Biomass, Furnace Oil, Gas & Chemical Process Applications

- Temp: Up to 250°C depending on scrubbing liquid and material selection

- Types of Scrubber: Venturi Type / Packed Bed / Spray Tower / Cyclonic Scrubber

- Types of Cleaning: Water Based / Chemical Based / Alkali Scrubbing System

- MOC: Carbon Steel ( IS2062) / FRP / SS304 / SS316 (If required FRP can be done)