CARBON FILTER- Adsorption System for Odour, VOC & Gas Phase Pollutant Removal

Product Overview:

Activated Carbon Filter removes odour, H₂S, mercaptans, solvents, and organic vapors from air/gas streams by adsorption. Microporous carbon granules trap odor molecules on their huge internal surface area. Used as final polishing stage after dust collectors or scrubbers where smell is the main issue.

Ideal For: STP odour, ETP exhausts, food processing, Pharma solvents, chemical fumes, kitchen exhaust, waste plants

Key Features:

- High Surface Area: 1 gram of carbon = 500-1500 m² internal area. Huge capacity to trap odor molecules.

- Broad Spectrum: Removes 100+ compounds – H₂S, mercaptans, amines, aldehydes, ketones, solvents, ozone.

- No Water/Chemical: Dry process, no effluent, no dosing pumps. Plug-and-play after commissioning.

- Low Pressure Drop: 100-150 mm wc only. Can run with existing AHU/fan in most cases.

- Impregnated Options: KOH, KI, or KMnO₄ impregnated carbon for H₂S, NH₃, formaldehyde, mercury.

- Modular Design: Multiple trays or cartridges for easy carbon change. No vessel entry needed.

- Polishing Stage: Achieves outlet odour <2 OU/M³ or non-detectable by human nose.

- Fire Safety: Thermal sensors + water spray for beds >5 M³ carbon. Prevents auto-ignition.

Technical Specifications:

- Capacity Range: 500 CMH to 2,00,000 CMH

- Type of Application: Odour Control /  Fume & Gas Filtration

- Temp: Up to 80°C to 120°C depending on carbon media

- Types of Carbon Media: Activated Carbon Granules / Pellets / Impregnated Carbon

- Types of Filtration: Single Bed / Multi Bed / Chemical Adsorption System

- MOC: Carbon Steel ( IS2062) / FRP / SS304 / SS316