COMPACT DUST COLLECTOR -

Product Overview:

Compact Dust Collector is a high-efficiency, small-footprint dust collection system using pleated cartridge filters. Designed for source capture, intermittent duty, and indoor installation, it replaces bulky bag filters where space is limited. Pulse jet cleaning provides continuous online operation with minimal compressed air.

Ideal For: Welding fumes, laser/plasma cutting, grinding, Pharma tablet press, Silo vent, food mixing, packaging lines, CNC machining.

Key Features:

- 50% Smaller Footprint: Pleated cartridges give 3x more area than bags in same volume. Fits between machines.

- Quick Cartridge Change: Top or front access door. No tools needed. Change in <5 minutes, no vessel entry.

- High Efficiency: PTFE membrane gives MERV 16, captures welding fumes, laser smoke, 0.1-1.0 micron dust.

- Low Compressed Air: Optimized venturi + diaphragm valve uses 30% less air than bag filters.

- Plug-and-Play: Comes with fan, controls, silencer, bin pre-wired. Just connect duct + power + air.

- Flame Retardant Media: For spark-generating apps like grinding. Prevents fire propagation.

- Down flow Pattern: Dirty air enters top, flows down. Heavy particles drop out, reduces filter loading.

Technical Specification:

- Capacity Range: 500 CMH to 10,000 CMH

- Type of Application: Dust Collection / Powder Handling / Fume Extraction Systems

- Temp: Up to 150°C with suitable filter media

- Types of Filter Bags/Cartridges: Polyester / PTFE Coated / Cellulose / Cartridge Type Filters

- Types of Cleaning: Pulse Jet

- MOC: Carbon Steel (IS2062) / SS304 / SS316