TELESCOPIC LOADING SPOUT-

Product Overview:

Telescopic Loading Spout is a retractable chute system for dust-free, spillage-free loading of dry bulk powders and granules into tankers, trucks, rail wagons, and ships. The spout extends to the material pile and retracts as level rises. Integrated dust extraction keeps emission <20 mg/NM³ during loading.

Ideal For: Cement, fly ash, flour, sugar, plastic granules, chemicals, grains, soda ash, carbon black

Key Features:

- Dust-Free Loading: Outer spout creates vacuum annulus. Dust pulled to integral filter or central dust collector. <20 mg/NM³ at truck hatch.

- Auto Level Control: Tilt probe or radar senses material pile. Spout retracts automatically to maintain seal. No operator needed.

- Abrasion Resistant: Hardox cones, ceramic liners, or replaceable PU cones for cement, clinker, sand. 3x life vs MS.

- Zero Spillage: Flexible outlet skirt with 360° seal sits on tanker hatch. No material escapes, no cleaning.

- Truck Friendly: Anti-collision sensor stops descent if truck moves. Prevents spout damage.

- Integral Filter Option: Self-cleaning cartridge on spout eliminates long ducting to central filter. Saves space.

- Hot Material Duty: Stainless steel cones + insulation + cooling ring for clinker up to 450°C.

Technical Specifications:

- Capacity Range: 50 TPH to 1000 TPH Material Loading Capacity

- Type of Application: Truck / Wagon / Ship Loading for Cement, Fly Ash, Clinker, Powder Materials, Food Grains

- Temp: Up to 200°C depending on material handled

- Types of Spout: Single Bellow / Double Bellow / Dust Controlled Loading Spout

- Types of Operation: Manual / Motorized / Automatic Telescopic System

- MOC: Carbon Steel (IS2062) / SS304 / SS316