FLOW CONTROL GATES-

Product Overview:

Flow Control Gate is a shut-off and regulating device installed below silos, hoppers, conveyors, and chutes to start, stop, or modulate flow of dry bulk solids. Available as slide gate, knife gate, or diverter type, it provides positive shut-off, variable discharge, and material routing with dust-tight sealing.

Ideal For: Cement, fly ash, grains, sugar, plastic granules, chemicals, sand, coal, minerals

Key Features:

- Positive Shut-Off: Knife edge cuts through material column. No leakage when closed, even with fine powders.

- Dust-Tight Design: Full perimeter seal + bonnet gasket. No dust emission to atmosphere.

- Wear Resistant: Hardox or ceramic lined blade and guides for abrasive materials like cement, sand, clinker.

- Modulating Control: V-notch blade or multi-blade louver for linear flow control 10-100% with actuator.

- Fail-Safe Options: Spring-close on air/power failure. Prevents silo emptying during emergency.

- Self-Cleaning: Blade scrapes seat on closing. No material buildup to prevent full closure.

- Low Headroom: Slim body design, 100-250mm height only. Fits under existing hoppers easily.

- Maintenance Friendly: Top-mounted actuator, replaceable blade, split frame for in-line service.

Technical Specifications:

- Capacity Range: 10 TPH to 500 TPH Material Handling Capacity

- Type of Application: Flow Control for Hoppers, Silos, Chutes & Conveying Systems

- Temp: Up to 400°C depending on material and sealing arrangement

- Types of Gates: Slide Gate / Flap Gate / Diverter Gate

- Types of Operation: Manual / Pneumatic / Motorized

- MOC: Carbon Steel (IS2062)