PNEUMATIC SLIDE GATES-

Product Overview:

Pneumatic Slide Gate is an air-cylinder operated shut-off device that uses a flat blade to open or close flow of dry bulk solids from silos, hoppers, and chutes. Fast 2-5 second actuation makes it ideal for batching, dosing, and process interlocking. Provides dust-tight sealing with positive material cut-off.

Ideal For: Batch weighing, truck loading, feeder isolation, filter discharge, pneumatic conveying systems

Key Features:

- Fast Operation: 2-5 sec full stroke vs 30-60 sec for motorized. Perfect for batching accuracy.

- Fail-Safe Action: Spring-return or air-reservoir closes gate on air/power failure. No silo emptying risk.

- Dust-Tight Seal: Full perimeter gasket + bonnet seal. <0.1% leakage, <20 mg/NM³ emission.

- Self-Cleaning Blade: Beveled edge scrapes seat clean on closing. No material jam or leakage.

- Low Profile: Body height 120-200mm only. Fits in tight spaces under existing equipment.

- Position Feedback: Magnetic reed switches for open/close status to PLC. No false signals.

- Maintenance Free: Only cylinder seals wear. Blade & body last 5+ years in cement duty.

- Modular Design: Bolted construction. Change blade/seal without removing gate from line.

Technical Specifications:

- Capacity Range: 10 TPH to 500 TPH Material Handling Capacity

- Type of Application: Dust Tight Flow Control for Hoppers, Silos & Pneumatic Conveying Systems

- Temp: Up to 250°C depending on sealing and actuator type

- Types of Gates: Slide Type / Knife Edge

- Types of Operation: Pneumatic Cylinder Operated / Solenoid Controlled Automation

- MOC: Carbon Steel ( IS2062)